E:TABLES:SRFMPTBL.DOC

A. Type I Criteria: Age, Origin, Landform, Material.

- Age of Surficial Material

H = Holocene (< 10,000 years old)

W = Wisconsin (ca. 89 to 10 ka)

I = Illinoian

P = Pleistocene Undifferentiated

EP = Early Pleistocene

MPl= Middle Pleistocene

LP = Late Pleistocene

Q = Quaternary Undifferentiated

CZ = Cenozoic Undifferentiated

Q-CZ(?) = Quaternary to Cenozoic?

MZ = Mesozoic (applied to bedrock)

PZ = Paleozoic (applied to bedrock)

2. Origin / Surficial Process

a = stream alluvium (normal flow)

sw = slackwater deposition

hcf = hyperconcentrated flow

c = colluvium  undifferentiated

co = collapse

cr = cryoturbation

ds = debris slide (viscous slurry flow)

df = debris flow

rf = rock fall or topple

r = residuum (in situ  regolith)

l = lacustrine deposit, undiff.

lb = lake-bottom deposit

ld = lacustrine deltaic

ls = lakeshore deposit (incl. beaches)

g = glaciofluvial, undifferentiated

go = glacial outwash

e = eolian

x = anthropogenic disturbance

f = artificial fill

rk = bedrock (continuous outcrop)

3. Landform Units

ch = channel

fp = floodplain (RI </= 2-3 yr)

lo = lobe

lv = levee

ox = oxbow, abandoned channel

t = terrace (above floodplain level)

f = fan

f-t = fan terrace

a = apron

n = nose

sl = side slope

h = hollow

bedrock = exposed bedrock

blanket = > 2 m of regolith

veneer = < 2m of regolith

ft = flow track (debris flows)

hm = hummocky topography

tls = talus deposits

rb = rock-block slide deposits

bk = block field

bs = boulder stream

pg = patterned ground

x = excavated, fill, disturbed ground

d = delta

du = dune

4. Material (Composition and Texture)

b = boulders (>256 mm)

bbd = very bouldery diamicton

bd = bouldery diamicton

c = cobbles (64-256 mm)

cd = cobbly diamicton

cy = clay

d = diamicton undifferentiated

ds = sandy matrix diamicton

dt = silty matrix diamicton

dy = clayey-matrix diamicton

g = gravel

l = loam

p = pebbles

pd = pebbly diamicton

s = sand

sg = mixed sand and gravel

st = silt

rk = bedrock (modify with lithology)

rs = rotten stone, saprolite

u = unkown

w = water

tr = travertine

tu = tufa

ma = marl

og = organic-rich sediment

B. Type II Criteria: 2-D Surface Features

bv = blind valley

ca = cave (human entry)

Active cave passage

Abandoned cave passage

dv = dry valley

kw = karst window

sk = sinkhole (doline)

skst = sinking stream

hs = headscar

tc = terracettes

ls = landslide scar undifferentiated

rs = rotational slide (slump) scar

ts = translational slide scar

rb = rock-block slide scar

wf = water fall

w = water, lake, reservoir

Spring

Seepage line

wt = wetland, undifferentiated

wh = wetland, heath

wm = wetland, marsh

ws = swamp

quarry (with highwall)

gravel pit

deep mine opening

strip mine (with highwall)

mine subsidence zone

rc = rock city

Scarp

Cliff

Meander scroll on floodplain

Lacustrine strandline

C. Type III Criteria: - Data Reference Points

Sandwhich symbols showing stratigraphy

Depth to bedrock (drilling or seismic data)

Minimum depth to bedrock (log data)

Test hole / boring

Well

RE = refusal (in test boring)

Hand-auger hole, shovel hole,

Fossil locality

Paleocurrent direction

Observation Point